Paxton Specification

Section Two

Net2 Readers

Edit as required – sections that are not required for a specific project can be removed, sections can be cut and merged with additional Paxton specifier documents for a complete/customised Access Control solution.

Key - 	[Square brackets] – Options. Delete brackets and turn off bold to include.

{Curly brackets} – Comments.

# SECTION 2

# ACCESS CONTROL – Net2 Readers

# GENERAL

## SUMMARY

### Section includes

#### Electronic Access Control

### Related sections

#### 28 13 00 Access Control

#### 28 13 33 Access Control Interfaces

### Products

### {Delete any that are not required for this project}

### {Choose to use either the Paxton product name or a general name for each}

#### [Proximity reader] [Paxton Proximity P series reader]

#### [Proximity keypad reader] [Paxton Proximity KP series keypad]

#### [Vandal proof proximity reader] [Paxton Vandal proof proximity reader]

#### [Proximity Panel mount reader] [Paxton Proximity panel mount reader]

#### [Proximity HID™ Panel mount reader] [Paxton Proximity HID™ panel mount reader]

#### [Metal proximity reader] [Paxton Metal proximity reader]

#### [Backbox proximity reader] [Paxton Backbox proximity reader]

#### [Mag-stripe reader] [Paxton CARDLOCK reader]

#### [Keypad] [Paxton TOUCHLOCK K series keypad]

#### [Stainless steel keypad] [Paxton Stainless steel TOUCHLOCK K series keypad]

#### [Architectural proximity reader] [Paxton Architectural proximity reader]

#### [Marine proximity reader] [Paxton Marine proximity reader]

#### [Universal proximity reader] [Paxton Universal proximity reader]

#### [Energy saving proximity reader] [Paxton Energy saving proximity reader]

#### [Long range proximity reader] [Paxton Long range proximity reader]

#### [Vandal resistant metal keypad] [Paxton TOUCHLOCK vandal resistant metal keypad]

#### [MIFARE® proximity reader] [Paxton MIFARE® P series proximity reader]

#### [GSM Access reader] [Paxton Net2 Caller ID reader]

#### [HID™ proximity reader] [Paxton HID™ proximity reader]

#### [MIFARE® proximity keypad reader] [Paxton MIFARE® KP proximity reader]

#### [Backbox MIFARE® proximity reader] [Paxton Backbox MIFARE® proximity reader]

#### [Backbox adaptor] [Paxton Backbox adaptor]

### System

#### The readers shall form a fundamental part of an Access Control System (ACS).

#### The ACS shall be scalable, allowing for additional readers [and other access control hardware] from the same manufacturer.

#### The readers shall be plug-and-play.

#### The ACS installed shall support plug-and-play readers.

## PROJECT REQUIREMENTS

### {Delete/Modify any that don’t apply to this project}

### A reader shall be installed next to each access point within the site to control and monitor its passage.

### Readers shall be installed next to entrance and exit of an area to provide anti-passback functionality.

### Each user shall be provided with a token.

#### The ACS installed shall support at minimum 50,000 unique token IDs.

### A reader shall be installed in conjunction with an ACU for arming/disarming an intruder alarm.

### Readers shall require a single cable which shall provide both power and communication.

### The readers combined with the ACS shall meet the access control requirements of the project.

### {Delete if necessary} The readers shall be installed into a site with an existing [Paxton] access control system.

### {Delete if necessary} The readers shall be installed with additional [Paxton] access control hardware and software to form a complete access control solution.

### Overall reader capability

#### The readers shall provide a method of identifying users.

#### Each reader shall be able to communicate the credential data to the ACS.

#### The ACS shall support at minimum 4 readers for each controlled access point.

#### Readers shall provide access granted and access denied feedback to the token holder.

### Components of the readers:

#### The reader, which shall provide the ACS with a user’s credentials.

#### The Access Control Unit (ACU), which shall provide the user database and processing to deny or allow access.

### The ACS shall provide the following as a minimum:

#### Access Control.

#### Door state monitoring (Door forced, Door left open).

#### Event reporting to the access control software.

#### License free Server and Client software and lifetime updates at no charge.

#### CCTV integration.

#### Site graphics.

#### Triggers and actions.

#### Intruder alarm integration.

#### Biometric integration.

#### A method by which 3rd party applications can be integrated via the use of an SDK or API

## DEFINITIONS

### Access Control System (ACS): The entire system as a whole, made up of hardware to lock/unlock an access point, readers to identify users, software/hardware to determine access control, and all firmware and additional hardware installed as part of the access control solution.

### Access Control Unit (ACU): An intelligent peripheral control unit that provides the interface between the Management and Monitoring Subsystem and the devices installed at the access portal for the purpose of restricting access and monitoring the portal status.

### AWG (American Wire Gauge): A unit of measurement for the diameter of wires.

### GSM (Global System for Mobile Communications): The standard to describe the protocols commonly used in mobile communications.

### IP: Internet Protocol incorporated into Microsoft Windows.

### LAN: Local area network.

### Open air: Without obstruction or interference.

### PC: Personal computer, used as the Central Station, workstations, and file servers.

### PoE: Power over Ethernet.

### Reader: A proximity reader or biometric reader that captures the credential information and passes it to the ACU for processing.

### Server: A PC that contains the database of users and access control settings, which runs the access control software.

### TCP: Transport Control Protocol incorporated into Microsoft Windows.

### Token: The credential issued to a person. This can be a PIN or a device containing an encoded number, used to determine if access will be granted or denied.

### USB: Universal Serial Bus.

### WAN: Wide area network.

### Windows: Operating system by Microsoft Corporation.

### Workstation: A PC with software that is configured for specific limited security system functions.

## ACCESS CONTROL DESCRIPTION

### General

#### Users shall be identified and processed through the means of:

##### Presenting a token to a reader.

##### Entering a unique PIN to a keypad.

##### Phone number via [GSM Access reader] [Paxton Net2 Caller ID reader].

##### Combination of the items above.

#### The system shall not use facility codes for card credentials.  Each token/credential shall have a unique 40 bit encryption for high security.

#### The system shall provide for unique card serial numbers, so the user will not need to determine the next sequence of cards to purchase.

### Access Control Software

#### The Access Control hardware shall be configurable using software provided by the manufacturer.

##### This software will have been included with the hardware at no additional cost.

##### Additional features sets shall be available with a licenced version of the software.

#### The software shall feature:

##### A graphical user interface to show pull-down menus and a menu tree format that complies with interface guidelines of Microsoft Windows operating system.

##### A mobile application for user administration, event reporting, door open and roll call.

###### The mobile application will be available for Android and iOS devices.

#### System license shall be for the entire system and shall include capability for future additions that are within the indicated system size limits specified in this Section. There shall be no license fee or yearly renewal fees.

#### Access shall be restricted using password-protected operator login.

#### The software at a minimum shall support the following operating systems:

##### Windows 10 Pro 64Bit

##### Windows 8 & Windows 8.1

##### Windows 7

##### Windows Vista

##### Windows Server 2008 & 2012

#### Number of PCs required

##### There shall not be a limit to the number of PCs that can run the configuration software. (Limits shall be placed based on the SQL licensing.)

##### There shall not be a limit to the number of mobile devices that can install and run the Android and iOS mobile application.

## PERFORMANCE REQUIREMENTS

### Equipment used shall be covered by a manufacturer’s warranty for a minimum of 5 years. The following aspects shall be covered:

#### Electrical

#### Electronic

#### Component

#### Mechanical

### Equipment used shall be provided with firmware upgrades from the manufacturer free of charge.

### Any change made in the software shall automatically be sent to the access control hardware when it is next online.

### Access control system capabilities shall be at a minimum:

#### 1000 access points

#### 50,000 users/tokens

## QUALITY ASSURANCE

### The ACS vendor shall offer a 5-year non-prorated warranty to cover the reader components and include all software upgrades.

## RELATED DOCUMENTS

### The ACS must interact with other physical parts of the facility and any construction either new or remodelled.

### When determining locations for placement of access control devices, the installer shall follow and be responsible for all appropriate building codes and laws concerning life safety and construction.

## COMPLIANCE

### {Remove any products that aren’t required for this project}

### {Remove any standards not relevant to the country of installation}

### [Proximity reader] [Paxton Proximity P series reader] shall comply with the following standards:

#### IPX7 for Moisture resistance {excluding screw connector product variations}

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### Industry Canada RSS-210 for Licence-exempt Radio Apparatus {excluding screw connector variations}

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

#### UL 294 for Access Control {excluding P38 & P200}

### [Proximity keypad reader] [Paxton Proximity KP series keypad] shall comply with the following standards:

#### IPX7 for Moisture resistance {excluding screw connector product variations}

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### Industry Canada RSS-210 for Licence-exempt Radio Apparatus {excluding Screw connector variations}

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

#### UL 294 for Access Control {excluding KP50}

### [Vandal proof proximity reader] [Paxton Vandal proof proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

#### ETSI EN 302 291 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Proximity Panel mount reader] [Paxton Proximity panel mount reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### Industry Canada RSS-210 for Licence-exempt Radio Apparatus

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Proximity HID™ Panel mount reader] [Paxton Proximity HID™ panel mount reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### Industry Canada RSS-210 for Licence-exempt Radio Apparatus

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

#### UL 294 for Access Control

### [Metal proximity reader] [Paxton Metal proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### Industry Canada RSS-210 for Licence-exempt Radio Apparatus

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

#### UL 294 for Access Control

### [Backbox proximity reader] [Paxton Backbox proximity reader] shall comply with the following standards:

#### IEC/EN 60950-1 for general safety

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Mag-stripe reader] [Paxton CARDLOCK reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

### [Keypad] [Paxton TOUCHLOCK K series keypad] shall comply with the following standards:

#### IPX7 for Moisture resistance {excluding screw connector product variations}

#### IEC/EN 60950-1 for general safety

#### UL 294 for Access Control

### [Stainless steel keypad] [Paxton Stainless steel TOUCHLOCK K series keypad] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### UL 294 for Access Control

### [Architectural proximity reader] [Paxton Architectural proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### Industry Canada RSS-210 for Licence-exempt Radio Apparatus

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Marine proximity reader] [Paxton Marine proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### Industry Canada RSS-210 for Licence-exempt Radio Apparatus

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Universal proximity reader] [Paxton Universal proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Energy saving proximity reader] [Paxton Energy saving proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

#### ETSI EN 302 291 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Long range proximity reader] [Paxton Long range proximity reader] shall comply with the following standards:

#### IPX6 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Vandal resistant metal keypad] [Paxton TOUCHLOCK vandal resistant metal keypad] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

### [MIFARE® proximity reader] [Paxton MIFARE® P series proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### ETSI EN 302 291 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [GSM Access reader] [Paxton Net2 Caller ID reader] shall comply with the following standards:

#### IEC/EN 60950-1 for general safety

### [HID™ proximity reader] [Paxton HID™ proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### FCC Part 15 Subpart C for Intentional Radiators

#### Industry Canada RSS-210 for Licence-exempt Radio Apparatus

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

#### UL 294 for Access Control

### [MIFARE® proximity keypad reader] [Paxton MIFARE® KP proximity reader] shall comply with the following standards:

#### IPX7 for Moisture resistance

#### IEC/EN 60950-1 for general safety

#### ETSI EN 302 291 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

### [Backbox MIFARE® proximity reader] [Paxton Backbox MIFARE® proximity reader] shall comply with the following standards:

#### IEC/EN 60950-1 for general safety

#### ETSI EN 300 330 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

#### ETSI EN 302 291 for Electromagnetic compatibility and Radio spectrum Matters (ERM)

## GENERAL REQUIREMENTS FOR FUNCTIONALITY

### Through use of reader, it shall be possible to read a user’s credential:

### {Remove any not relevant to the project}

#### A credential holder shall be able to present their token to the reader to gain valid access.

#### Presentation of an invalid credential shall not allow access.

#### A credential holder shall be able to enter their PIN into a keypad to gain valid access.

#### Entering of an invalid PIN shall not allow access.

### Visual feedback shall be provided upon presentation of a credential.

#### Feedback shall indicate if the credential is valid or invalid for the intended action.

### Audible feedback shall be provided upon presentation of a credential.

#### Feedback shall indicate if the credential is valid or invalid for the intended action.

### Readers shall be connected to the ACU:

#### Readers shall communicate with the ACU.

#### Readers shall be powered by the ACU.

### The access control software shall record door entry events.

#### An event shall be generated for the following:

##### Following a valid credential being read, when the door is unlocked.

##### Following an invalid credential being read, when the door is not unlocked.

##### When the door is forced.

##### When the door is left open.

#### All events shall be time and date stamped.

#### All events shall contain which door they relate to.

#### All events shall be communicated with the Server in real-time

## GENERAL REQUIREMENTS FOR COMMUNICATIONS

### ACU to Reader

#### Readers shall communicate with an ACU over any of the following or technical equivalent cable:

##### Belden 9540

###### 8 core screened cable.

###### 18 AWG

###### Maximum length 100m (328')

##### Belden 9538

###### 8 core screened cable.

###### 22 AWG

###### Maximum length 25m (82')

#### All surface mount readers shall be provided with 5m (16.4') Belden 9540 cable at no additional cost.

### No manual addressing shall be required.

# PRODUCTS

## MANUFACTURERS

### {Delete if necessary} Acceptable manufacturer: Paxton

#### Email address: {Delete options which are not required}

##### {UK} [support@paxton.co.uk]

##### {FR} [support@paxtonaccess.fr]

##### {US} [supportUS@paxton-access.com]

##### {DE} [verkauf@paxton-gmbh.de]

##### {NL} [support@paxton-benelux.com]

#### Telephone number:

##### {UK} [01273 811011]

##### {FR} [01 57 32 93 56]

##### {US} [877.438.7298]

##### {DE} [0251 2080 6900]

##### {NL} [076 3333 999]

#### Skype:

##### {UK} [Paxton.support]

##### {FR} [Paxton.benelux.support]

##### {US} [usaPaxton.support]

##### {DE} [Paxton.gmbh.support]

### {Delete if necessary} Substitutions: Not permitted.

### {Delete if necessary} The components of the ACS shall be available from a single source manufacturer to assure compatibility of products.

### The manufacturer of the reader shall also provide an Access Control System.

### The manufacturer of the reader shall also provide a Door Entry System.

### Substitution Limitations

#### It shall be possible to install a reader into an existing [Paxton] ACS.

#### It shall be possible to install a reader as a functional replacement to an existing reader.

## SPECIFIC REQUIREMENTS FOR TOKENS

### The manufacturer of the reader shall be able to supply Paxton HiTag2 125KHz tokens.

#### The supplied tokens shall contain an authentication method to deter the copying and unauthorised use of tokens.

## SPECIFIC REQUIREMENTS FOR [PROXIMITY READER] [PAXTON PROXIMITY P SERIES READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

###### HID 125KHz {may require activation}

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

###### Hands free token

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 120 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F).

### Housing

#### The item shall be stylish and modern.

#### The item shall have a similar look and feel on all product variations.

#### There shall be no visible fixings on the item.

#### It shall be possible to install the reader surface mount.

##### A backbox adaptor shall be available from the ACS manufacturer for the ‘large’ product variant for flush mount installation.

### Product variations

#### The item shall be available in Small [- P38]

##### The dimensions shall not exceed:

###### A width of 38mm (1.5")

###### A height of 78mm (3.1")

###### A depth of 12.5mm (0.5")

##### Token read range for each token type shall be at minimum:

###### Keyfob – 40mm (1.6")

###### Token/ISO card – 60mm (2.4")

###### Watchprox – 30mm (1.2")

###### Hands free token – 850mm (33.5")

#### The item shall be available in Medium [- P50]

##### The dimensions shall not exceed:

###### A width of 50mm (2")

###### A height of 100mm (3.9")

###### A depth of 15mm (0.6")

##### Token read range for each token type shall be at minimum:

###### Keyfob – 50mm (2")

###### Token/ISO card – 80mm (3.1")

###### Watchprox – 35mm (1.4")

###### Hands free token – 1100mm (43.3")

#### The item shall be available in Large [- P75]

##### The dimensions shall not exceed:

###### A width of 75mm (3")

###### A height of 143mm (5.6")

###### A depth of 16mm (0.6")

##### Token read range for each token type shall be at minimum:

###### Keyfob – 60mm (2.4")

###### Token/ISO card – 100mm (3.9")

###### Watchprox – 40mm (1.6")

###### Hands free token – 1500mm (59.1")

#### The item shall be available in Extra-large [- P200]

##### The dimensions shall not exceed:

###### A width of 200mm (7.9")

###### A height of 200mm (7.9")

###### A depth of 15mm (0.6")

##### Token read range for each token type shall be at minimum:

###### Keyfob – 100mm (3.9")

###### Token/ISO card – 200mm (7.9")

###### Watchprox – 50mm (2")

###### Hands free token – 2500mm (98.4")

#### Each product variant shall be available with an option of clip on covers.

##### At minimum, the covers shall be available in the following colours:

###### Black

###### White

###### Grey

###### Silver

###### Blue

## SPECIFIC REQUIREMENTS FOR [PROXIMITY KEYPAD READER] [PAXTON PROXIMITY KP SERIES KEYPAD] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

###### HID 125KHz {may require activation}

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

###### Hands free token

#### The item shall contain a keypad.

##### The following formats of credential shall be supported

###### PIN

###### Code

#### Identification shall consist of:

##### Token only

##### Token + PIN

##### Token + Code

##### PIN only

##### Code only

##### Token or PIN

##### Token or Code

##### Token, PIN or Code

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 180 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F)

### Housing

#### The item shall be stylish and modern.

#### The item shall have a similar look and feel on all product variations.

#### There shall be no visible fixings on the item.

#### It shall be possible to install the reader surface mount.

##### A backbox adaptor shall be available from the ACS manufacturer for the ‘large’ product variant for flush mount installation.

### Product variations

#### The item shall be available in Medium [- KP50]

##### The dimensions shall not exceed:

###### A width of 50mm (2")

###### A height of 100mm (3.9")

###### A depth of 15mm (0.6")

##### Token read range for each token type shall be at minimum:

###### Keyfob – 50mm (2")

###### Token/ISO card – 80mm (3.1")

###### Watchprox – 35mm (1.4")

###### Hands free token – 1100mm (43.3")

#### The item shall be available in Large [- KP75]

##### The dimensions shall not exceed:

###### A width of 75mm (3")

###### A height of 143mm (5.6")

###### A depth of 16mm (0.6")

##### Token read range for each token type shall be at minimum:

###### Keyfob – 60mm (2.4")

###### Token/ISO card – 100mm (3.9")

###### Watchprox – 40mm (1.6")

###### Hands free token – 1500mm (59.1")

#### Each product variant shall be available with an option of clip on covers.

##### At minimum, the covers shall be available in the following colours:

###### Black

###### White

###### Grey

###### Silver

###### Blue

## SPECIFIC REQUIREMENTS FOR [VANDAL PROOF PROXIMITY READER] [PAXTON VANDAL PROOF PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

###### HID 125KHz {UK/EU only - may require activation}

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

##### Token read range for each token type shall be at minimum:

###### Keyfob – 40mm (1.6")

###### Token/ISO card – 80 mm (3.1")

###### Watchprox – 20mm (0.8")

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 100 mA.

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### There shall be no visible fixings on the item.

#### The item shall be split into 2 parts:

##### The tube, which shall contain the reader’s main components.

###### This shall be installed hidden from direct access.

###### This shall be secure and not accessible from the unsecure side of the access point.

##### The cap, which shall be used as an indication of where the tube is located.

###### This shall be installed on the unsecure side of the access point.

### Dimensions

#### The dimensions for the tube shall not exceed:

##### A diameter of 20mm (0.8")

##### A height of 60mm (2.4")

#### The dimensions for the cap shall not exceed:

##### A width of 30mm (1.2")

##### A height of 48mm (1.9")

##### A depth of 9mm (0.4")

#### Product Variants

##### At minimum, the caps shall be available in the following colours:

###### Black

###### White

###### Grey

###### Solid blue

###### Clear blue

###### Green

###### Yellow

## SPECIFIC REQUIREMENTS FOR [PROXIMITY PANEL MOUNT READER] [PAXTON PROXIMITY PANEL MOUNT READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Features

#### Designed for use in audio/video entry panels.

#### Fits neatly behind the industry standard size square aperture.

##### This shall be 40mm square.

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

###### HID 125KHz {UK/EU only - may require activation}

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 110 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F)

### Housing

#### The reader shall be installed into an audio/video entry panel.

#### The item shall contain a clear polycarbonate cover to give a neat finish.

#### The reader shall be vandal resistant.

#### The reader shall be moisture resistant.

### Dimensions

#### The item dimensions shall be:

##### A width of 58mm (2.3")

##### A height of 58mm (2.3")

##### A depth of 18mm (0.7")

## SPECIFIC REQUIREMENTS FOR [PROXIMITY HID™ PANEL MOUNT READER] [PAXTON PROXIMITY HID™ PANEL MOUNT READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Features

#### Designed for use in audio/video entry panels.

#### Fits neatly behind the industry standard size square aperture.

##### This shall be 40mm square.

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### 125 kHz HID

###### EM4100

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 130 mA.

### Display

#### The item shall house 3 LEDs.

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F)

### Dimensions

#### The item dimensions shall be:

##### A width of 58mm (2.3")

##### A height of 58mm (2.3")

##### A depth of 17mm (0.7")

## SPECIFIC REQUIREMENTS FOR [METAL PROXIMITY READER] [PAXTON METAL PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

###### HID 125KHz {UK/EU only - may require activation}

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

##### Token read range for each token type shall be at minimum:

###### Keyfob – 40mm (1.6")

###### Token/ISO card – 50mm (2")

###### Watchprox – 10mm (0.4")

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 110 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F)

### Housing

#### The housing shall be metal.

#### The housing shall be vandal resistant.

### Dimensions

#### The item dimensions shall not exceed:

##### A width of 76mm (3")

##### A height of 76mm (3")

##### A depth of 27mm (1.1")

#### Product Variants

##### At minimum, the item shall be available in the following finishes:

###### Chrome

###### Satin Chrome

## SPECIFIC REQUIREMENTS FOR [BACKBOX PROXIMITY READER] [PAXTON BACKBOX PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

##### The above credential technology shall be read simultaneously.

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

##### Token read range for each token type shall be at minimum:

###### Keyfob – 60mm (2.4")

###### Token/ISO card – 100mm (3.9")

###### Watchprox – 40mm (1.6")

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 140 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

#### It shall be possible to disable audible feedback

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### The item shall be stylish and modern.

#### There shall be no visible fixings on the item.

#### The item shall be designed for internal use.

#### It shall be possible to install the reader:

##### Flush mount

###### The reader shall fit into standard single gang UK electrical backboxes.

##### Surface mount

### Dimensions

#### The dimensions shall not exceed:

##### A width of 86mm (3.4")

##### A height of 86mm (3.4")

##### A depth of 16mm (0.6")

#### Product Variants

##### At minimum, the item shall be available in the following colours:

###### Black

###### White

## SPECIFIC REQUIREMENTS FOR [MAG-STRIPE READER] [PAXTON CARDLOCK READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a magnetic stripe reader.

##### At a minimum, the following token technology shall be supported:

###### Track 2 ABA

##### The following formats of credential shall be supported:

###### Mag-stripe card

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 90 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### The housing shall be Satin Chrome.

### Dimensions

#### The dimensions shall not exceed:

##### A width of 34mm (1.3")

##### A height of 89mm (3.5")

##### A depth of 35mm (1.4")

## SPECIFIC REQUIREMENTS FOR [KEYPAD] [PAXTON TOUCHLOCK K SERIES KEYPAD] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a keypad.

##### The following formats of credential shall be supported

###### PIN

###### Code

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 120 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

#### The keys shall be backlit for ease of use in low light applications.

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +50.8°F)

### Housing

#### The item shall be stylish and modern.

#### The item shall have a similar look and feel on all product variations.

#### There shall be no visible fixings on the item.

#### It shall be possible to install the reader surface mount.

##### A backbox adaptor shall be available from the ACS manufacturer for the ‘large’ product variant for flush mount installation.

### Product variations

#### The item shall be available in Medium [- K50]

##### The dimensions shall not exceed:

###### A width of 50mm (2")

###### A height of 100mm (3.9")

###### A depth of 15mm (0.6")

#### The item shall be available in large [- K75]

##### The dimensions shall not exceed:

###### A width of 75mm (3")

###### A height of 143mm (5.6")

###### A depth of 16mm (0.6")

#### Each product variant shall be available with an option of clip on covers.

##### At minimum, the covers shall be available in the following colours:

###### Black

###### White

## SPECIFIC REQUIREMENTS FOR [STAINLESS STEEL KEYPAD] [PAXTON STAINLESS STEEL TOUCHLOCK K SERIES KEYPAD] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a keypad.

##### The following formats of credential shall be supported

###### PIN

###### Code

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 120 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

#### The keys shall be backlit for ease of use in low light applications.

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +50.8°F)

### Housing

#### The item shall be stylish and modern.

#### The item shall be constructed from brushed stainless steel.

#### The buttons and bezel shall have a satin chrome stainless steel finish.

#### There shall be no visible fixings on the item.

#### It shall be possible to install the reader surface mount.

##### A backbox adaptor shall be available from the ACS manufacturer for the ‘large’ product variant for flush mount installation.

### Product variations

#### The item shall be available in Medium [- K50]

##### The dimensions shall not exceed:

###### A width of 50mm (2")

###### A height of 100mm (3.9")

###### A depth of 15mm (0.6")

#### The item shall be available in large [- K75]

##### The dimensions shall not exceed:

###### A width of 75mm (3")

###### A height of 143mm (5.6")

###### A depth of 16mm (0.6")

## SPECIFIC REQUIREMENTS FOR [ARCHITECTURAL PROXIMITY READER] [PAXTON ARCHITECTURAL PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

###### HID 125KHz {may require activation}

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

###### Hands free token

##### Token read range for each token type shall be at minimum:

###### Keyfob – 25mm (1")

###### Token/ISO card – 40mm (1.6")

###### Watchprox – 15mm (0.6")

###### Hands free token – 400mm (15.7")

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 140 mA.

### Display

#### The item shall house 3 LEDs.

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### All product variants shall be stylish and modern.

#### The item shall have a similar look and feel on all product variations.

#### There shall be no visible fixings on the item.

#### The item shall consist of 2 parts:

##### Reader

##### Insert

#### There shall be a choice of finishes for the reader to suit the site aesthetics.

#### There shall be a choice of inserts to suit the site aesthetics.

### Dimensions

#### The reader dimensions shall not exceed:

##### A width of 70mm (2.8")

##### A height of 110mm (4.3")

##### A depth of 35mm (1.4")

#### The insert dimensions shall be at minimum:

##### A width of 61mm (2.4")

##### A height of 102mm (4")

##### A depth of 6mm (0.2")

### Product variations

#### The reader shall be available in a range of finishes, including the following as a minimum:

##### Matt black

##### Gunmetal grey

##### Satin chrome

#### A range of inserts shall be available, including the following as a minimum:

##### Stone

##### Glass

## SPECIFIC REQUIREMENTS FOR [MARINE PROXIMITY READER] [PAXTON MARINE PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

###### HID 125KHz {may require activation}

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

###### Hands free token

##### Token read range for each token type shall be at minimum:

###### Keyfob – 30mm (1.2")

###### Token/ISO card – 40mm (1.6")

###### Watchprox – 20mm (0.8")

###### Hands free token – 400mm (15.7")

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 220 mA.

### Display

#### The item shall house a stylish LED display.

#### The LED display shall be easily viewable and not obstructed on presentation of a token.

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +70°C (-31°F to +158°F)

### Housing

#### The item shall be stylish and modern.

#### The item shall have a similar look and feel on all product variations.

#### There shall be no visible fixings on the item.

#### The item shall be manufactured from marine grade stainless steel.

### Dimensions

#### The item dimensions shall not exceed:

##### A diameter of 106mm (4.2")

##### A depth of 16mm (0.6")

### Product variations

#### The item shall be available in Stainless steel finish.

## SPECIFIC REQUIREMENTS FOR [UNIVERSAL PROXIMITY READER] [PAXTON UNIVERSAL PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

###### PAC

##### The above credential technology shall be read simultaneously.

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

##### Token read range for each token type shall be at minimum:

###### Keyfob – 40mm (1.6")

###### Token/ISO card – 60mm (2.4")

###### Watchprox – 30mm (1.2")

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 120 mA.

### Display

#### The item shall house 3 LEDs.

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### The item shall be stylish and modern.

#### The item shall have a similar look and feel on all product variations.

#### There shall be no visible fixings on the item.

### Dimensions

#### The dimensions shall not exceed:

##### A width of 38mm (1.5")

##### A height of 78mm (3.1")

##### A depth of 12.5mm (0.5")

### Product variations

#### The item shall be available with an option of clip on covers.

##### At minimum, the covers shall be available in the following colours:

###### Black

###### White

##### These shall be available from the manufacturer free of charge.

## SPECIFIC REQUIREMENTS FOR [ENERGY SAVING PROXIMITY READER] [PAXTON ENERGY SAVING PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton Hitag2 125 kHz

###### Hitag1 / Hitag2

###### MIFARE 1K

###### MIFARE 4K

###### MIFARE Ultralight / C

###### MIFARE DESFire

###### SmartMX

###### R5

###### EM4100

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 170 mA.

### Display

#### The item shall house 3 LEDs.

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### The item shall be stylish and modern.

#### There shall be no visible fixings on the item.

### Dimensions

#### The dimensions shall not exceed:

##### A width of 61mm (2.4")

##### A height of 105mm (4.1")

##### A depth of 17mm (0.7")

## SPECIFIC REQUIREMENTS FOR [LONG RANGE PROXIMITY READER] [PAXTON LONG RANGE PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### EM4100

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

###### Hands free token

##### Token read range for each token type shall be at minimum:

###### Keyfob – 40mm (1.6")

###### Token/ISO card – 80mm (3.1")

###### Watchprox – 10mm (0.4")

###### Hands free token – 5000mm (196.8")

##### It shall be possible to change the 2.4GHz frequency channel to improve system performance.

###### At minimum there shall be 16 selectable channels.

### Power Supply

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 1.1 A.

### Display

#### The item shall feature an LED display for visual feedback and indications.

#### The LED display shall be visible in sunlight.

#### The LED display shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F)

### Housing

#### The item shall be stylish and modern.

#### There shall be no visible fixings on the item.

#### The housing shall be robust to withstand the external environment.

### Dimensions

#### The dimensions shall not exceed:

##### A width of 220mm (8.7")

##### A height of 220mm (8.7")

##### A depth of 135mm (5.3")

## SPECIFIC REQUIREMENTS FOR [VANDAL RESISTANT METAL KEYPAD] [PAXTON TOUCHLOCK VANDAL RESISTANT METAL KEYPAD] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a keypad.

##### The following formats of credential shall be supported

###### PIN

###### Code

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 150 mA.

### Display

#### Keypad keys shall be backlit.

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### The item shall be vandal resistant.

#### The item shall be constructed from metal.

### Dimensions

#### The dimensions shall not exceed:

##### A width of 89mm (3.5")

##### A height of 143mm (5.6")

##### A depth of 28mm (1.1")

## SPECIFIC REQUIREMENTS FOR [MIFARE® PROXIMITY READER] [PAXTON MIFARE® P SERIES PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton HiTag2 125KHz

###### Hitag2

###### EM4100

###### MIFARE 1K

###### MIFARE 4K

###### MIFARE Ultralight / C

###### MIFARE DESFire

###### SmartMX

###### R5

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

###### Hands free token

##### Token read range for each token type shall be at minimum:

###### Paxton HiTag2 125KHz

###### Keyfob – 30mm (1.2")

###### Token – 80mm (3.1")

###### Watchprox – 20mm (0.8")

###### Hands free token – 850mm (33.5")

###### EM4100

###### Keyfob – 50mm (2")

###### Card – 60mm (2.4")

###### Proxdisc – 30mm (1.2")

###### Hitag

###### Hitag2 ISO card – 70mm (2.8")

###### MIFARE

###### 1K – 50mm (2")

###### 4K – 40mm (1.6")

###### Ultralight / C – 25mm (1")

###### DESFire – 20 mm (0.8")

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 170 mA.

### Display

#### The item shall house 3 LEDs.

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F)

### Housing

#### The item shall be stylish and modern.

#### The item shall have a similar look and feel on all product variations.

#### There shall be no visible fixings on the item.

### Dimensions

#### The item dimensions shall not exceed:

##### A width of 50mm (2")

##### A height of 100mm (3.9")

##### A depth of 15mm (0.6")

### Product variations

#### The item shall be available with an option of clip on covers.

##### At minimum, the covers shall be available in the following colours:

###### Black

###### White

###### Grey

###### Silver

###### Blue

##### These covers shall be available free of charge.

## SPECIFIC REQUIREMENTS FOR [GSM ACCESS READER] [PAXTON NET2 CALLER ID READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall support Global System for Mobile communication (GSM).

##### The reader shall allow any phone to be used as a credential.

##### Assuming both the reader and credential device have GSM signal, the range shall be unlimited,

#### The item shall allow for an additional token reader to be connected.

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 500 mA.

### Display

#### The item shall house LEDs.

#### The LEDs shall indicate the following states:

##### Item is powered

##### Item is connected to mobile phone network (GSM network)

##### Item is receiving data from a mobile phone

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### The item shall be designed for internal installation.

### Dimensions

#### The item dimensions shall not exceed:

##### A width of 200mm (7.9")

##### A height of 200mm (7.9")

##### A depth of 78mm (3.1")

## SPECIFIC REQUIREMENTS FOR [HID™ PROXIMITY READER] [PAXTON HID™ PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### 125 kHz HID

###### EM4100

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 130 mA.

### Display

#### The item shall house 3 LEDs.

#### The LEDs shall indicate the following events:

##### Valid credential– access granted

##### Invalid credential– access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F)

### Product variations

#### The item shall be available in 2 variations:

##### Mullion reader

###### Credential read range

###### Read range shall be at minimum 85mm (3.3")

###### Housing

The item shall be stylish and modern.

There shall be no visible fixings on the item.

The item shall be available in a variety of colours.

##### Metal reader

###### Credential read range

###### Read range shall be at minimum 40mm (1.6")

###### Housing

The housing shall be vandal resistant.

###### The item shall be available in a satin chrome finish.

### Dimensions

#### The Mullion reader dimensions shall not exceed:

##### A width of 45mm (1.8")

##### A height of 100mm (3.9")

##### A depth of 20mm (0.8")

#### The Metal reader dimensions shall not exceed:

##### A width of 76mm (3")

##### A height of 76mm (3")

##### A depth of 27mm (1.1")

## SPECIFIC REQUIREMENTS FOR [MIFARE® PROXIMITY KEYPAD READER] [PAXTON MIFARE® KP PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### MIFARE 1K

###### MIFARE 4K

###### MIFARE Ultralight / C

###### MIFARE DESFire / EV1

###### SmartMX

###### R5

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

#### The item shall contain a keypad.

##### The following formats of credential shall be supported

###### PIN

###### Code

#### It shall be possible to require a combination of PIN and token for identification.

#### It shall be possible to require a combination of Code and token for identification.

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 12V DC.

#### Current consumption shall not exceed 200 mA.

### Display

#### The item shall house 3 LEDs

##### 1 x Green LED

##### 1 x Amber LED

##### 1 x Red LED

#### The LEDs shall indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall meet the required temperature standards for an external product

##### The item shall operate reliably within the temperature range of -35°C to +66°C (-31°F to +150.8°F)

### Housing

#### The item shall be stylish and modern.

#### There shall be no visible fixings on the item.

#### The housing shall be metal.

#### The housing shall be vandal resistant.

#### It shall be possible to install the reader surface mount.

##### A backbox adaptor shall be available from the ACS manufacturer for flush mount installation.

### Dimensions

#### The dimensions shall not exceed:

##### A width of 75mm (3")

##### A height of 143mm (5.6")

##### A depth of 16mm (0.6")

## SPECIFIC REQUIREMENTS FOR [BACKBOX MIFARE® PROXIMITY READER] [PAXTON BACKBOX MIFARE® PROXIMITY READER] {Delete as required}

### {Remove this section if this product is not required for the project}

### Credential Identification

#### The item shall contain a proximity reader.

##### At a minimum, the following token technology shall be supported:

###### Paxton Hitag2 125KHz

###### Hitag1 / Hitag2

###### EM4100

###### MIFARE 1K

###### MIFARE 4K

###### MIFARE Ultralight / C

###### MIFARE DESFire / EV1

###### SmartMX

###### R5

##### The above credential technology shall be read simultaneously.

##### The following formats of credential shall be supported:

###### Keyfob

###### Token

###### ISO card

###### Watchprox

###### Hands free token

###### Proxdisc

##### Token read range for each token type shall be at minimum:

###### Paxton HiTag2 125KHz

###### Keyfob – 60mm (2.4")

###### Token – 125mm (4.9")

###### Watchprox – 50mm (2")

###### Hands free token – 800mm (31.5")

###### Hitag

###### Hitag1 ISO – 50mm (2")

###### Hitag2 ISO – 125mm (4.9")

###### EM4100

###### Keyfob – 60mm (2.4")

###### Token – 50mm (2")

###### Proxdisc – 80mm (3.1")

###### MIFARE

###### 1K – 20mm (0.8")

###### 4K – 20mm (0.8")

###### Ultralight / C – 10mm (0.4")

###### DESFire / EV1 – 10 mm (0.4")

### Power Supply

#### The item shall be powered by the ACU.

#### Operating voltage shall not exceed 14V DC.

#### Current consumption shall not exceed 140 mA.

### Display

#### The item shall house LEDs.

#### The LEDs shall indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Audio

#### The item shall contain a sounder for audible feedback

#### A tone shall sound to indicate the following events:

##### Valid credential read – access granted

##### Invalid credential read – access denied

###### Invalid permissions

###### Unknown credential

###### Barred credential

### Temperature

#### The item shall operate reliably within the temperature range of -20°C to +55°C (-4°F to +131°F)

### Housing

#### The item shall be stylish and modern.

#### There shall be no visible fixings on the item.

#### The item shall be designed for internal use.

#### It shall be possible to install the reader flush mount:

###### The reader shall fit into standard single gang UK electrical backboxes.

###### The reader shall fit into standard single gang US electrical backboxes.

###### The reader shall fit into standard single gang EU electrical backboxes.

### Dimensions

#### The dimensions shall not exceed:

##### A width of 87mm (3.4")

##### A height of 125mm (4.9")

##### A depth of 18mm (0.7")

## SPECIFIC REQUIREMENTS FOR [BACKBOX ADAPTOR] [PAXTON BACKBOX ADAPTOR] {Delete as required}

### {Remove this section if this product is not required for the project}

### Features

#### This adaptor shall allow the following readers to be flush mounted: {Delete as required}

##### [Proximity reader - Large] [Paxton Proximity P series reader – P75]

##### [Proximity keypad reader - Large] [Paxton Proximity KP series keypad – KP75]

##### [Keypad - Large] [Paxton TOUCHLOCK K series keypad – K75]

##### [Stainless steel keypad - Large] [Paxton Stainless steel TOUCHLOCK K series keypad – K75]

#### The adaptor shall be mounted to standard UK backboxes.

### Housing

##### The item shall be available in:

###### Black

###### White

### Dimensions

#### The dimensions shall not exceed:

##### A width of 75mm (3")

##### A height of 142mm (5.6")

##### A depth of 8mm (0.3")

# END OF SECTION